Report Title Goes Here

A Subtitle or Study Description, If Needed

Andres Henrique de Souza Palomo

Fire Simualtion - BUW

August 2026

# Abstract

Write a 150-250 word summary here: the problem, your method, the key results, and the main conclusion. This paragraph should stand alone.

# Table of Contents

Abstract	1

Table of Contents	2

List of Figures	2

List of Tables	2

1. Introduction	3

2. Methods	3

2.1 Sub-section Example	3

3. Results	3

4. Discussion	3

5. Conclusion	3

References	4

## List of Figures

Figure : Description of the first figure.	3

Figure : Description of the second figure.	3

## List of Tables

Table 1: Fuel components characterization.	3

Table 2: Surgate fuel characterization.	3

# 1. Introduction

This study is a semi blind simulation of a full-scale compartment fire experiment conducted in December 2025 within an empty residential building in Leipzig, Germany. It is considered a semi blind experiment because important characteristics required for conducting the simulations are provided, such as the fuel type, its mass, the mass loss curve, and the relevant characteristics of the apartment, but not the results and a few assumptions have to be made. The simulated data allows us to understand dangerous situations in a safe manner and can be compared with the data collected during the real experiment. Such validation studies are important because they validate the use of fire simulations to improve fire safety, estimate evacuation times, and assess possible structural damage.

The experiment involves a liquid pool fire fueled by a commercial C7 hydrocarbon mixture in an unfurnished apartment. The test compartment consists of three interconnected rooms (F1, F2, F3) and a corridor (FC), giving a total floor area of approximately 50.67 m² and a room height of 2.54 m. All internal doors were removed, and during the 20 minutes pre-ventilation phase the windows remained closed, mechanical ventilation was absent, and only background leakage contributed to air exchange. This provided undisturbed fire development conditions. The fire source consisted of a square steel pan measuring 160 mm × 160 mm with a depth of 100 mm containing 150 g of the C7 fuel, positioned at the center of room F1. Several measuring devices were placed throughout the apartment, including thermocouples and smoke detectors. This allows the comparison between the simulated results and the data collected during the real experiment.

The objective of this report is to present a detailed computational fluid dynamics (CFD) simulation of this scenario, with the following quantities of interest:

- Gas temperature - thermocouples in F1, F2 and F3 at eight heights (0.6–2.5 m), plus a ceiling thermocouple above the pan in F1. The template uses QUANTITY='THERMOCOUPLE' to match the radiation-affected bead response of the measured thermocouples.

- Optical extinction - extinction coefficient σ [1/m] at the optical measurement locations, reference wavelength 638 nm.

- Smoke alarm actuation - optical smoke alarms (DIN EN 14604) at the ceiling in F2, FC and F3.

- Tenability - assessed by the participant in F1, F2, FC and F3.

# 2. Methods

This section is meant to guide the reader on how the simulation was set up in accordance to the description of experiment. First its describe how the fuel for the simulation is defined, then how the modeling of the apartment is done, including the measurements devices and finally the mesh.

## 2.1 Surrogate fuel characterization

In this simulation it is used the commercial C7-UVCB hydrocarbon mixture labelled “Heptan” as fuel. Since this fuel is not pre-defined in the FDS dataset, its description must be given with parameters that FDS uses to conduct the simulation.

The following approximation is used as the composition by compound class (wt.%, derived from the measured molar fractions):

- n-alkanes: approximately 35 wt.% (predominantly n-heptane)

- iso-alkanes: approximately 15 wt.%

- cyclic alkanes (naphthenes): approximately 45 wt.% (predominantly methylcyclohexane and cyclohexane)

- aromatic content: very low

The fuel should therefore not be interpreted as chemically pure n-heptane. For this FDS modelling, the following C7 surrogate fuel was assumed:

Table 1: Fuel components characterization.

| Category | Component | Mass [%] | Formula | M [g/mol] | Heat of combustion [kJ/g] |  |  | Yield [kg/kg] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔHₜ | ΔHch | ΔHrad | CO | Soot |
| n-alkenes | n-Heptane | 35% | C₇H₁₆ | 100 | 44,6 | 41,2 | 13,6 | 0.012 | 0.042 |
| Iso-alkanes | Methylhexane | 15% | C₇H₁₆ | 100 | 44,6 | 41,1 | 14,2 | 0.012 | 0.037 |
| cyclic alkanes | Methyl-cyclohexane | 25% | C₇H₁₄ | 98 | 43,4 | 38,5 | 15,0 | 0.018 | 0.054 |
| cyclic alkanes | Cyclohexane | 20% | C6H12 | 84 | 43,8 | 39,9 | 15,3 | 0.016 | 0.049 |
| aromatic content | Toluene | 5% | C₇H₈ | 92 | 39,7 | 27,0 | 15,9 | 0.064 | 0.173 |

All of this values are taken from the Table A.40 from the Fire Engineering Handbook, in which ΔHt is the total heat of combustion, ΔHch is the chemical heat and ΔHrad is the radiative heat. With composition it’s define the following Surgate Fuel in FDS:

Table 2: Surgate fuel characterization.

| Component | Formula | M [g/mol] | ΔHₜ [kJ/kg] | ΔHch [kJ/kg] | ΔHrad [kJ/kg] | Radiative fraction | CO yield [kg/kg] | Soot yield [kg/kg] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Surgate C7-UVCB | C7H14.66 | 95.9 | 43895 | 39540 | 14495 | 0.33 | 0,0169 | 0,0522 |

These values are inputs in FDS. Chemical heat is considered as the value HEAT_OF_COMBUSTION, since it is the heat released in an ideal complete combustion. Using Radiative heat and Total heat of combustion RADIATIVE_FRACTION can be computed

RADIATIVEFRACTION= ΔHradΔHt =0.33

## 2.2 Apartment modeling

The apartment has to be model as closely as possible to the reality. Since heat will be transferred from the hot fluid flowing around the rooms, different material will contribute differently in the room temperature. Also, leakage is expected and can be modeled in FDS.

### 2.2.1 Wall modeling

The set-up of the experiment considers the walls to be solid clay brick masonry. The corresponding material in the Fire Engineering Handbook is “Building brick, common”, which is defined with the following properties:

Table 3: Wall characterization.

| Substance | Temperature [ºC] | k [W/m· ºC] | ρ [kg/m3] | C [kJ/kg· ºC] | α [m2/s] |
| --- | --- | --- | --- | --- | --- |
| Building brick, common | 20 | 0.69 | 1,600 | 0.84 | 5.2×10-7 |

### 2.2.1 Windows

The set-up of the experiment considers the to be double glazing, with timber frames in F1 and F2, PVC frame in F3. They are always closed throughout the pre-ventilation phase is defined with the following properties:

Table 3: Windows characterization.

| Substance | Temperature [ºC] | k [W/m· ºC] | ρ [kg/m3] | C [kJ/kg· ºC] | α [m2/s] |
| --- | --- | --- | --- | --- | --- |
| Building brick, common | 20 | 0.69 | 1,600 | 0.84 | 5.2×10-7 |

### 2.2.1 Floor and ceiling

The set-up of the experiment considers the to be double glazing, with timber frames in F1 and F2, PVC frame in F3. They are always closed throughout the pre-ventilation phase is defined with the following properties:

Table 3: Windows characterization.

| Substance | Temperature [ºC] | k [W/m· ºC] | ρ [kg/m3] | C [kJ/kg· ºC] | α [m2/s] |
| --- | --- | --- | --- | --- | --- |
| Building brick, common | 20 | 0.69 | 1,600 | 0.84 | 5.2×10-7 |

### 2.2.1 Leakage

The set-up of the experiment considers the to be double glazing, with timber frames in F1 and F2, PVC frame in F3. They are always closed throughout the pre-ventilation phase is defined with the following properties:

Table 3: Windows characterization.

| Substance | Temperature [ºC] | k [W/m· ºC] | ρ [kg/m3] | C [kJ/kg· ºC] | α [m2/s] |
| --- | --- | --- | --- | --- | --- |
| Building brick, common | 20 | 0.69 | 1,600 | 0.84 | 5.2×10-7 |

# 3. Results

In this section an analytical look on the result of the simulation is taken to validate the simulation and get the “Quantities of interest” proposed by the study. These Quantities of interest are:

- Gas temperature — thermocouples in F1, F2 and F3 at eight heights (0.6–2.5 m), plus a ceiling thermocouple above the pan in F1. The template uses QUANTITY='THERMOCOUPLE' to match the radiation-affected bead response of the measured thermocouples.

- Optical extinction — extinction coefficient σ [1/m] at the optical measurement locations, reference wavelength 638 nm.

- Smoke alarm actuation — optical smoke alarms (DIN EN 14604) at the ceiling in F2, FC and F3.

- Tenability — assessed by the participant in F1, F2, FC and F3.

Those quantities are relevant to understand Smoke alarm actuation

As shown in Figure 1, replace this placeholder box below with your actual image (Insert > Picture), then apply the 'Caption' style to its caption line so the numbering stays automatic.

[ Insert Figure Here ]

Figure 1: Description of the first figure.

A second figure, referenced as Figure 2, follows the same pattern.

[ Insert Figure Here ]

Figure 2: Description of the second figure.

# 4. Discussion

Interpret the results, compare them against expectations or literature, and discuss limitations.

# 5. Conclusion

Summarize the main findings and, if relevant, suggest future work.

# References

- Author, A. (Year). Title of the work. Journal or Publisher, Volume(Issue), pages.

- Author, B. (Year). Title of another work. Journal or Publisher.